F5 BIG-IP SSL Policy Enhancement Project Plan

Project Name: F5 BIG-IP SSL Policy Enhancement
Version: 1.0
Date: 2026-01-21
Project Owner: Bronson So

EXECUTIVE SUMMARY

Business Need: This project addresses critical security vulnerabilities by eliminating weak TLS 1.0/1.1 protocols for external traffic and standardizing cipher suites to meet PCI DSS 3.2.1 and NIST SP 800-52 requirements. The phased approach minimizes business disruption while improving our security posture.

Key Benefits:

Risk Reduction: Mitigate POODLE, BEAST, and other TLS 1.0/1.1 vulnerabilities

Compliance: Achieve PCI DSS 3.2.1 compliance for external-facing services

Standardization: Unified security configuration across 32 SSL profiles

Operational Efficiency: Simplified management with template-based profiles

Investment: 74 person-hours over 17 business days
Risk Level: MEDIUM (controlled through phased deployment and rollback)

1. Project Overview

1.1 Background

The company needs to enhance SSL/TLS security policies on F5 BIG-IP devices, implementing differentiated encryption standards based on traffic origin:

External Traffic: Enforce TLS 1.2 and above, disable TLS 1.0 and 1.1

Internal/Trusted Domain Traffic: Allow TLS 1.0, 1.1, 1.2, and 1.3

1.2 Business Justification & ROI

| Driver | Business Impact | Success Metric |
| --- | --- | --- |
| Security Risk Mitigation | Reduce attack surface by eliminating vulnerable protocols | External TLS 1.0/1.1 traffic reduced to 0% |
| Compliance Requirements | Meet PCI DSS 3.2.1 (effective June 2018) requirement for TLS 1.2+ | Pass quarterly ASV scans without TLS-related findings |
| Reputation Protection | Maintain customer trust with A+ SSL Labs ratings | All external sites achieve A or A+ rating |

1.3 Objectives

Enforce TLS 1.2+ with modern cipher suites for external traffic

Standardize all SSL profiles based on 3 security templates (Legacy, Standard, High)

Eliminate weak/obsolete ciphers (RC4, 3DES, export-grade, CBC-mode where possible)

Implement intelligent SSL selection based on source IP using existing Data Groups

Establish monitoring and alerting for legacy TLS usage in internal network (optional)

1.4 Scope

Included:

F5 BIG-IP SSL Profile configuration and cipher standardization

Tls 1.0, 1.1 external traffic capture on F5 BIG-IP or mirror port on switch

iRule development for dynamic SSL selection

TLS versions and cipher suite standardization across prod and uat environments

Excluded:

Application-level TLS configuration modifications

SSL policy adjustments on non-F5 devices

Hardware SSL offloading performance tuning

Client-side application updates

1.5 Stakeholder Management (RACI Matrix)

| Stakeholder | Role | Responsibilities | Communication Frequency |
| --- | --- | --- | --- |
| Network Team | Responsible | Define cipher standards, validate compliance, Implement, test, deploy changes | Daily during deployment |
| Application Teams | Consulted | Validate application compatibility | Pre/post deployment |
| Service Desk | Informed | Handle user reports, escalate issues | Daily briefings during rollout |
| IT Team Head & Business Unit Heads | Informed | Awareness of potential impacts | Pre-deployment notification |

2. Current Environment Analysis

2.1 F5 Device Information

| Item | Information |
| --- | --- |
| Device Names | F5 BIG-IP |
| Software Version | 17.5.1.3 Build 0.192.19 Engineering Hotfix |
| Deployment Environment | Production (prod) and UAT (uat) environments |
| Partitions | common, prod, uat |

2.2 Current SSL/TLS Configuration

2.2.1 Overview

32 client SSL profiles with clear separation between Production (prod) and UAT (uat) environments (appendix I)

Hierarchical design using template profiles for consistent security policies (appendix II)

Two primary certificate chains: hkec.chain.crt (prod) and u_hkec.chain.crt (uat)

2.2.2 SSL Profile Configuration

| Name | Security Level | No. of Profile | TLS Version | Cipher Posture |
| --- | --- | --- | --- | --- |
| High | High | 11 | 1.2 + | Cipher posture: GCM-only (AES-GCM) and/or CHACHA20 only Forward secrecy: PFS-capable suites only (ECDHE/DHE) No CBC suites detected No SHA1 / RC4 / 3DES / NULL / EXPORT / MD5 indicators detected |
| Standard | Medium | 7 | 1.2 + | Cipher posture: includes CBC-mode suites (typically SHA256) May still include GCM suites, but not strictly GCM-only No SHA1 / weak suite indicators detected |
| Legacy | Low | 14 | 1.0 + | Cipher posture: allows deprecated SHA1 suites and/or non‑PFS RSA key exchange suites (compatibility) Often includes CBC suites and legacy-compatible RSA suites Highest compatibility, lowest security posture among the four classes |

2.2.3 SSL Profile Usage Pattern (TBC)

| Services | Network | SSL Profile |
| --- | --- | --- |
| AI Services | External | High |
|  | Internal | Low |
| EDB Applications | External | High |
|  | Internal | Low |
| Project Sites | External | Standard |
|  | Internal | Low |
| General Services | External | Standard |
|  | Internal | Low |
| Management | External | High |
|  | Internal | Low |

2.3 Identified Network and IP Ranges

Internal Network:

Class A network

From 10.0.0.0 to 10.5.x.x

using 22,23,24,26 subnets

External Network:

202.8.88.0/22

210.5.185.66/32

220.241.141.1/32

20.187.162.128/32

20.187.187.28/32

2.4 Other Services

The following services are not included in this project

| Services |  |
| --- | --- |
| SMTP | ltm data-group internal prod/mbox_smtp_nosnat { 10.0.0.116/32 { } 10.0.0.123/32 { }} |
| LDAPS | ltm monitor ldap prod/hkec_ldap { destination *:* # Could be port 389 (LDAP) or 636 (LDAPS)} |
| MySQL | ltm monitor mysql prod/proxysql_6033 { destination *:6033} |
| FTPS | ltm monitor tcp uat/u_hkec_ftps { destination *.ftp} |

3. Technical Implementation Plan

3.1 Proposal Options

Option 1: Enable TLS 1.0/1.1 Logging on F5 BIG-IP

Description: Enable logging on F5 BIG-IP devices to capture TLS 1.0/1.1 external traffic. Data is generated directly on the load balancer and exported for analysis.

Approach: Gradual rollout from a few selected IPs to full IP coverage in both UAT and Production.

Duration: 1–3 months (TBC)

Option 2: Capture Traffic via Switch Mirror Port (Wireshark/WorldShark)

Description: Configure a mirror port on the switch to capture TLS 1.0/1.1 traffic. Packet captures are analysed using Wireshark/WorldShark.

Approach: Mirror selected IPs and expand gradually across UAT and Production.

Duration: 1–3 months (TBC)

Option 3: Apply SSL Client Profile and iRule on F5

Description: Deploy an SSL client profile combined with an iRule to detect and handle TLS 1.0/1.1 connections.

Approach: Start with a small number of IPs, expand in phases to full UAT and Production.

3.2 Phase 1: Preparation & Analysis

| Task | Description | Owner | Timeline | Status |
| --- | --- | --- | --- | --- |
| 1.1 | Audit current SSL Profiles | Network Team | Day 1-2 | Pending |
| 1.2 | Identify network and IPs using SSL profile | Network Team | Day 1-2 | Pending |
| 1.3 | Create test plan | Network Team | Day 2-3 | Pending |

3.2 Phase 2: Development & Testing

| Task | Description | Owner | Timeline | Status |
| --- | --- | --- | --- | --- |
| 2.1 | Create external SSL Profile (TLS 1.2+) in permissive mode | Network Team | Day 4-5 | Pending |
| 2.2 | Develop iRule for dynamic SSL selection | Network Team | Day 5-6 | Pending |
| 2.3 | Backup UAT configuration Deploy in UAT environment | Network Team | Day 6-8 | Pending |

3.3 Phase 3: Deployment

| Task | Description | Owner | Timeline | Status |
| --- | --- | --- | --- | --- |
| 3.1 | Schedule deployment window | Project Manager | Day 11 | Pending |
| 3.2 | Deploy to UAT | Network Team | Day 12 | Pending |
| 3.3 | Execute test plan | Network Team | Day 12 | Pending |
| 2.4 | Validate internal application compatibility | Application Teams | Day 7-9 | Pending |
| 3.4 | Deploy to production | Network Team | Day 13 | Pending |
| 2.5 | External security validation | Security Team | Day 9-10 | Pending |
| 3.5 | Post-deployment monitoring | Network Team | Day 13-14 | Pending |

3.4 Phase 4: Verification & Documentation

| Task | Description | Owner | Timeline | Status |
| --- | --- | --- | --- | --- |
| 4.1 | SSL Labs testing (external) | Security Team | Day 15 | Pending |
| 4.2 | Internal application validation | Application Teams | Day 15-16 | Pending |
| 4.3 | Update network documentation | Network Team | Day 16-17 | Pending |
| 4.4 | Project closure & lessons learned | Project Manager | Day 18 | Pending |

4. Comprehensive Risk Assessment

4.1 Risk Register

| Risk ID | Description | Probability | Impact | Mitigation | Owner |
| --- | --- | --- | --- | --- | --- |
| R-001 | Critical application fails after TLS upgrade | Medium | High | 1. Comprehensive UAT testing 2. Application compatibility matrix 3. Business-hour deployment for critical apps | App Teams |
| R-002 | Performance degradation with new ciphers | Low | Medium | 1. Performance baseline before change 2. CPU monitoring during rollout 3. Hardware SSL acceleration enabled | Network Team |
| R-003 | Legacy partner systems cannot connect | Medium | High | 1. Partner notification 60 days prior 2. Exception process for validated partners 3. Temporary workaround VIP | Partner Team |
| R-004 | Rollback fails during emergency | Low | Critical | 1. Test rollback procedure in UAT 2. Configuration backups verified 3. Secondary rollback path (device reboot) | Network Team |
| R-005 | Certificate compatibility issues | Medium | High | 1. Certificate audit before deployment 2. Ensure SHA-256 certificates 3. Certificate chain validation | Security Team |

5. Staged Deployment Strategy

This may not be needed with the three scheduled windows. But in case it is needed, below is the recommended order:

Low-traffic production VIPs (10%)

Medium-traffic VIPs (40%)

High-traffic VIPs (remaining)

5.1 Rollback Plan

markdown

1. **Immediate Rollback (0-15 minutes)**

- Remove iRule from VIP

- Reapply original SSL profile

- Verify service restoration

2. **Extended Rollback (15-60 minutes)**

- Restore from configuration backup

- Reload configuration

- Full service validation

3. **Communication During Rollback**

- Service desk: Immediate notification

- Stakeholders: Within 30 minutes

- Users: Service restoration notice only

4. **Post-Rollback Analysis**

- Root cause analysis within 24 hours

- Revised deployment plan within 48 hours

6. Success Criteria

6.1 Technical Matrix

✅ Security: 0% external TLS 1.0/1.1 traffic (measured weekly)

✅ Performance: <5% CPU increase on F5 devices

✅ Compliance: 100% of external VIPs pass PCI ASV scans

✅ Operational: All 31 profiles migrated to 3 templates

6.2 Business Matrix

✅ Availability: Zero unplanned downtime during business hours

✅ User Impact: <5 support tickets related to TLS changes

✅ Financial: No compliance fines or audit findings

✅ Strategic: Foundation for TLS 1.3 deployment established

7. Resource Requirements

7.1 Team Members

Project Manager: 10 hours

Network Engineers: 70 hours (1 engineer × 70 hours)

Security Analyst: 8 hours

Application Support: 16 hours

7.2 Tools & Resources

F5 BIG-IP CLI/GUI access

SSL Labs testing tool

Network monitoring tools

Change management system access

8. Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Project Sponsor |  |  |  |
| Network Manager |  |  |  |
| Security Manager |  |  |  |
| Change Advisory Board |  |  |  |

Appendix I

There are 32 ssl profiles across Common, prod and uat

## prod/ai_edcity_all_crt

ltm profile client-ssl prod/ai_edcity_all_crt {
    app-service none
    cert-key-chain {
        ai_edcity_all_crt_hkec.chain_0 {
            cert prod/ai_edcity_all_crt
            chain prod/hkec.chain.crt
            key prod/ai_edcity_all_crt
        }
    }
    defaults-from prod/hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/edcity_all_crt

ltm profile client-ssl prod/edcity_all_crt {
    app-service none
    cert-key-chain {
        edcity_all_crt_hkec.chain_0 {
            cert prod/edcity_all_crt
            chain prod/hkec.chain.crt
            key prod/edcity_all_crt
        }
    }
    defaults-from prod/hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/edcity_edb_all_crt

ltm profile client-ssl prod/edcity_edb_all_crt {
    app-service none
    cert-key-chain {
        edcity_edb_all_crt_hkec.chain_0 {
            cert prod/edcity_edb_all_crt
            chain prod/hkec.chain.crt
            key prod/edcity_edb_all_crt
        }
    }
    defaults-from prod/hkec_TLSv12-nokRSA
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/edcity_edb_all_crt_high

ltm profile client-ssl prod/edcity_edb_all_crt_high {
    app-service none
    cert-key-chain {
        edcity_edb_all_crt_hkec.chain_0 {
            cert prod/edcity_edb_all_crt
            chain prod/hkec.chain.crt
            key prod/edcity_edb_all_crt
        }
    }
    defaults-from prod/hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/edcity_proj_all_crt

ltm profile client-ssl prod/edcity_proj_all_crt {
    app-service none
    cert-key-chain {
        edcity_proj_all_crt_hkec.chain_0 {
            cert prod/edcity_proj_all_crt
            chain prod/hkec.chain.crt
            key prod/edcity_proj_all_crt
        }
    }
    defaults-from prod/hkec_TLSv12-nokRSA
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_TLSv12-nokRSA

ltm profile client-ssl prod/hkec_TLSv12-nokRSA {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-ECDSA-AES128-SHA
256,ECDHE-RSA-AES128-GCM-SHA256,ECDHE-RSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-SHA25
6,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-SHA256,DHE-RS
A-AES256-SHA256
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## prod/hkec_TLSv12-nokRSA-noCBC

ltm profile client-ssl prod/hkec_TLSv12-nokRSA-noCBC {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-GCM-S
HA256,ECDHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA
384
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## prod/hkec_all_crt

ltm profile client-ssl prod/hkec_all_crt {
    app-service none
    cert-key-chain {
        hkec_all_crt_hkec.chain_0 {
            cert prod/hkec_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_all_crt
        }
    }
    defaults-from prod/hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_all_crt_TLSv12-nokRSA

ltm profile client-ssl prod/hkec_all_crt_TLSv12-nokRSA {
    app-service none
    cert-key-chain {
        hkec_all_crt_hkec.chain_0 {
            cert prod/hkec_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_all_crt
        }
    }
    defaults-from prod/hkec_TLSv12-nokRSA
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_all_crt_high

ltm profile client-ssl prod/hkec_all_crt_high {
    app-service none
    cert-key-chain {
        hkec_all_crt_hkec.chain_0 {
            cert prod/hkec_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_all_crt
        }
    }
    defaults-from prod/hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_all_crt_sadm

ltm profile client-ssl prod/hkec_all_crt_sadm {
    app-service none
    cert-key-chain {
        hkec_all_crt_hkec.chain_0 {
            cert prod/hkec_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_all_crt
        }
    }
    defaults-from prod/hkec_TLSv12-nokRSA-noCBC
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_all_crt_wmng

ltm profile client-ssl prod/hkec_all_crt_wmng {
    app-service none
    cert-key-chain {
        hkec_all_crt_hkec.chain_0 {
            cert prod/hkec_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_all_crt
        }
    }
    defaults-from prod/hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_default

ltm profile client-ssl prod/hkec_default {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-ECDSA-AES128-SHA
256,ECDHE-RSA-AES128-GCM-SHA256,ECDHE-RSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-SHA25
6,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-SHA256,DHE-RS
A-AES256-SHA256,DHE-RSA-AES128-SHA,DHE-RSA-AES256-SHA,AES128-GCM-SHA256,AES256-GCM
-SHA384,AES128-SHA256,AES256-SHA256,AES128-SHA,AES256-SHA
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## prod/hkec_edb_all_crt_high

ltm profile client-ssl prod/hkec_edb_all_crt_high {
    app-service none
    cert-key-chain {
        hkec_edb_all_crt_hkec.chain_0 {
            cert prod/hkec_edb_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_edb_all_crt
        }
    }
    defaults-from prod/hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_high

ltm profile client-ssl prod/hkec_high {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-GCM-S
HA256,ECDHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA
384
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## prod/hkec_proj_all_crt

ltm profile client-ssl prod/hkec_proj_all_crt {
    app-service none
    cert-key-chain {
        hkec_proj_all_crt_hkec.chain_0 {
            cert prod/hkec_proj_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_proj_all_crt
        }
    }
    defaults-from prod/hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkec_star_all_crt

ltm profile client-ssl prod/hkec_star_all_crt {
    app-service none
    cert-key-chain {
        hkec_star_all_crt_hkec.chain_0 {
            cert prod/hkec_star_all_crt
            chain prod/hkec.chain.crt
            key prod/hkec_star_all_crt
        }
    }
    defaults-from prod/hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## prod/hkrc_all_crt

ltm profile client-ssl prod/hkrc_all_crt {
    app-service none
    cert-key-chain {
        hkrc_all_crt_hkec.chain_0 {
            cert prod/hkrc_all_crt
            chain prod/hkec.chain.crt
            key prod/hkrc_all_crt
        }
    }
    defaults-from prod/hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_ai_edcity_all_crt

ltm profile client-ssl uat/u_ai_edcity_all_crt {
    app-service none
    cert-key-chain {
        u_ai_edcity_all_crt_u_hkec.chain_0 {
            cert uat/u_ai_edcity_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_ai_edcity_all_crt
        }
    }
    defaults-from uat/u_hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_edcity_all_crt

ltm profile client-ssl uat/u_edcity_all_crt {
    app-service none
    cert-key-chain {
        u_edcity_all_crt_u_hkec.chain_0 {
            cert uat/u_edcity_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_edcity_all_crt
        }
    }
    defaults-from uat/u_hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_TLSv12-nokRSA

ltm profile client-ssl uat/u_hkec_TLSv12-nokRSA {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-ECDSA-AES128-SHA
256,ECDHE-RSA-AES128-GCM-SHA256,ECDHE-RSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-SHA25
6,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-SHA256,DHE-RS
A-AES256-SHA256
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## uat/u_hkec_all_crt

ltm profile client-ssl uat/u_hkec_all_crt {
    app-service none
    cert-key-chain {
        u_hkec_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_all_crt
        }
    }
    defaults-from uat/u_hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_all_crt_TLSv12-nokRSA

ltm profile client-ssl uat/u_hkec_all_crt_TLSv12-nokRSA {
    app-service none
    cert-key-chain {
        u_hkec_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_all_crt
        }
    }
    defaults-from uat/u_hkec_TLSv12-nokRSA
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_all_crt_high

ltm profile client-ssl uat/u_hkec_all_crt_high {
    app-service none
    cert-key-chain {
        u_hkec_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_all_crt
        }
    }
    defaults-from uat/u_hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_default

ltm profile client-ssl uat/u_hkec_default {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-ECDSA-AES128-SHA
256,ECDHE-RSA-AES128-GCM-SHA256,ECDHE-RSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-SHA25
6,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-SHA256,DHE-RS
A-AES256-SHA256,DHE-RSA-AES128-SHA,DHE-RSA-AES256-SHA,AES128-GCM-SHA256,AES256-GCM
-SHA384,AES128-SHA256,AES256-SHA256,AES128-SHA,AES256-SHA
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## uat/u_hkec_edb_all_crt_high

ltm profile client-ssl uat/u_hkec_edb_all_crt_high {
    app-service none
    cert-key-chain {
        u_hkec_edb_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_edb_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_edb_all_crt
        }
    }
    defaults-from uat/u_hkec_high
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_high

ltm profile client-ssl uat/u_hkec_high {
    app-service none
    cert-key-chain {
        default {
            cert Common/default.crt
            key Common/default.key
        }
    }
    cipher-group none
    ciphers
ECDHE-ECDSA-AES128-GCM-SHA256,ECDHE-ECDSA-AES256-GCM-SHA384,ECDHE-RSA-AES128-GCM-S
HA256,ECDHE-RSA-AES256-GCM-SHA384,DHE-RSA-AES128-GCM-SHA256,DHE-RSA-AES256-GCM-SHA
384
    defaults-from Common/clientssl
    inherit-ca-certkeychain true
    inherit-certkeychain true
    options { dont-insert-empty-fragments no-tlsv1.3 no-dtlsv1.2 no-sslv3 }
}

## uat/u_hkec_proj_all_crt

ltm profile client-ssl uat/u_hkec_proj_all_crt {
    app-service none
    cert-key-chain {
        u_hkec_proj_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_proj_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_proj_all_crt
        }
    }
    defaults-from uat/u_hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_star_all_crt

ltm profile client-ssl uat/u_hkec_star_all_crt {
    app-service none
    cert-key-chain {
        u_hkec_star_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_star_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_star_all_crt
        }
    }
    defaults-from uat/u_hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_uat_all_crt

ltm profile client-ssl uat/u_hkec_uat_all_crt {
    app-service none
    cert-key-chain {
        u_hkec_uat_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_uat_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_uat_all_crt
        }
    }
    defaults-from uat/u_hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkec_uat_all_crt_TLSv12-nokRSA

ltm profile client-ssl uat/u_hkec_uat_all_crt_TLSv12-nokRSA {
    app-service none
    cert-key-chain {
        u_hkec_uat_all_crt_u_hkec.chain_0 {
            cert uat/u_hkec_uat_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkec_uat_all_crt
        }
    }
    defaults-from uat/u_hkec_TLSv12-nokRSA
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

## uat/u_hkrc_all_crt

ltm profile client-ssl uat/u_hkrc_all_crt {
    app-service none
    cert-key-chain {
        u_hkrc_all_crt_u_hkec.chain_0 {
            cert uat/u_hkrc_all_crt
            chain uat/u_hkec.chain.crt
            key uat/u_hkrc_all_crt
        }
    }
    defaults-from uat/u_hkec_default
    inherit-ca-certkeychain true
    inherit-certkeychain false
}

Appendix II

Profiles are grouped into 4 security levels using their effective cipher lists and protocol/cipher indicators.

Level 1 (Maximum): GCM/CHACHA-only AND PFS-only (ECDHE/DHE) AND no CBC AND no RSA key-exchange ciphers.

Level 2 (High): PFS-only and no CBC / no RSA key-exchange ciphers, but cipher list may be less constrained than Level 1.

Level 3 (Medium): CBC present OR RSA key-exchange ciphers present, but no SHA1/RC4/3DES/NULL/EXPORT/MD5 indicators.

Level 4 (Standard/Legacy): SHA1 present OR weak ciphers (RC4/3DES/NULL/EXPORT/MD5) OR SSLv2/SSLv3 explicitly enabled.

# LEVEL 1: MAXIMUM SECURITY

├── prod/hkec_TLSv12-nokRSA-noCBC

│   ├── Only GCM-mode ciphers

│   ├── No CBC-mode ciphers detected

│   ├── PFS-only (ECDHE/DHE) — no RSA key-exchange suites detected

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

└── prod/hkec_all_crt_sadm

├── prod/hkec_high

│   ├── Only GCM-mode ciphers

│   ├── No CBC-mode ciphers detected

│   ├── PFS-only (ECDHE/DHE) — no RSA key-exchange suites detected

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

├── prod/ai_edcity_all_crt

├── prod/edcity_edb_all_crt_high

├── prod/hkec_all_crt_high

└── prod/hkec_edb_all_crt_high

├── uat/u_hkec_high

│   ├── Only GCM-mode ciphers

│   ├── No CBC-mode ciphers detected

│   ├── PFS-only (ECDHE/DHE) — no RSA key-exchange suites detected

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

├── uat/u_ai_edcity_all_crt

├── uat/u_hkec_all_crt_high

└── uat/u_hkec_edb_all_crt_high

# LEVEL 2: HIGH SECURITY

├── Common/do-not-remove-without-replacement

│   ├── No CBC-mode ciphers detected

│   ├── PFS-only (ECDHE/DHE) — no RSA key-exchange suites detected

│

└── Profiles inheriting this baseline:

└── (none)

# LEVEL 3: MEDIUM SECURITY

├── prod/hkec_TLSv12-nokRSA

│   ├── Includes GCM-mode ciphers

│   ├── Includes CBC-mode ciphers (consider Lucky13/BEAST exposure)

│   ├── PFS-only (ECDHE/DHE) — no RSA key-exchange suites detected

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

├── prod/edcity_edb_all_crt

├── prod/edcity_proj_all_crt

└── prod/hkec_all_crt_TLSv12-nokRSA

├── uat/u_hkec_TLSv12-nokRSA

│   ├── Includes GCM-mode ciphers

│   ├── Includes CBC-mode ciphers (consider Lucky13/BEAST exposure)

│   ├── PFS-only (ECDHE/DHE) — no RSA key-exchange suites detected

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

├── uat/u_hkec_all_crt_TLSv12-nokRSA

└── uat/u_hkec_uat_all_crt_TLSv12-nokRSA

# LEVEL 4: STANDARD / LEGACY

├── prod/hkec_default

│   ├── Includes GCM-mode ciphers

│   ├── Includes CBC-mode ciphers (consider Lucky13/BEAST exposure)

│   ├── Includes RSA key-exchange ciphers (no forward secrecy for those suites)

│   ├── Includes SHA1-based ciphers (compatibility / weak)

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

├── prod/edcity_all_crt

├── prod/hkec_all_crt

├── prod/hkec_all_crt_wmng

├── prod/hkec_proj_all_crt

├── prod/hkec_star_all_crt

└── prod/hkrc_all_crt

├── uat/u_hkec_default

│   ├── Includes GCM-mode ciphers

│   ├── Includes CBC-mode ciphers (consider Lucky13/BEAST exposure)

│   ├── Includes RSA key-exchange ciphers (no forward secrecy for those suites)

│   ├── Includes SHA1-based ciphers (compatibility / weak)

│   ├── Protocol options: TLSv1.3 disabled, SSLv3 disabled

│   └── Preferred cipher observed: ECDHE-RSA-AES256-GCM-SHA384

│

└── Profiles inheriting this baseline:

├── uat/u_edcity_all_crt

├── uat/u_hkec_all_crt

├── uat/u_hkec_proj_all_crt

├── uat/u_hkec_star_all_crt

├── uat/u_hkec_uat_all_crt

└── uat/u_hkrc_all_crt